Configuration Steps:

You first need to put your PC on a network that can reach the device IP 10.115.32.10. A practical way is to connect the Pi directly to your PC with an Ethernet cable and temporarily give your PC a compatible Ethernet address.

1. Connect the Pi directly to your PC using Ethernet.

This method assumes 10.115.32.10 is assigned to the Pi’s Ethernet interface, rather than its Wi-Fi interface. A USB-to-Ethernet adapter works if your PC lacks an Ethernet port.

2. Temporarily configure your PC’s Ethernet adapter.

On your Windows PC:

- Press Win + R, enter ncpa.cpl, and press Enter.

- Right-click the Ethernet adapter connected to the Pi → Properties.

- Select Internet Protocol Version 4 (TCP/IPv4) → Properties.

- Record its existing settings, then enter:

This assumes the Pi uses a /24 subnet mask (255.255.255.0). If its mask differs, match its actual subnet. Leave your PC’s Wi-Fi settings unchanged.

3. Open PowerShell as Administrator on your PC:

Enable remote connection

Net Start WinRM

Test the connection

Test-NetConnection 10.115.32.10 -Port 5985

Verify TcpTestSuccessed = True

Enter Trusted Host List.

Set-Item WSMan:\localhost\client\TrustedHosts – value 10.115.32.10

4. Start PowerSell Session

Enter-PSSession -Computer 10.115.32.10 -Credential 10.115.32.10\Administrator

You are now connected and logged in to the device.

6. Display current network interfaces

netsh interface ipv4 show config

7. Set Static IP

netsh interface ip set address “Ethernet” static 10.115.32.10 255.255.255.0 10.115.32.1

8. Display new network interface settings

netsh interface ipv4 show config